STRUCTURED ROBOTICS RESEARCH DOCUMENT

Wheel-Leg Hybrid Robot — Simulation, Control & Learning Pipeline

Clean, ordered, practical guide for end-to-end development

# 1. Project Goal

A two-wheeled legged robot (wheel-leg hybrid) ka complete simulation → control → learning pipeline banana.

Core principle: Pehle physics aur control sahi banao, baad mein RL aur Sim-to-Real.

# 2. Final Recommended Stack

Fixed stack — 5–6 different simulators install karke confuse mat hona.

| Order | Software / Tool | Purpose | Kab Use Karna Hai |
| --- | --- | --- | --- |
| 1 | Fusion 360 | Mechanical CAD design | Day 1 |
| 2 | Python + VS Code + Git | Coding + version control | Day 1 |
| 3 | MuJoCo ★ | Main physics simulator | Phase 2 se |
| 4 | NumPy + SciPy + Matplotlib | Math, optimization, plots | Phase 3 se |
| 5 | PyTorch + Gymnasium | Reinforcement Learning | Phase 9 ke baad |
| Later | ROS 2 | Real robot integration | Real hardware ke time |
| Later | Isaac Sim / Isaac Lab | Large-scale RL + Domain Randomization | MuJoCo solid hone ke baad |

Primary Physics Engine = MuJoCo. Isaac Sim / Isaac Lab ko abhi first simulator mat banao.

# 3. Core Pipeline

Main Simulation & Control Flow:

Fusion 360 (CAD)  →  MJCF Model (robot.xml)  →  MuJoCo Physics Engine

→  Sensor Simulation (IMU + Encoder + Contact)

→  State Estimation  →  Controller (PID → LQR → MPC)

→  Motor Commands  →  MuJoCo Feedback Loop

Later (after basics are solid):

MuJoCo → Basic RL (PPO) → Isaac Lab → Domain Randomization → Real Robot

# 4. Phase-wise Exact Execution Plan

## Phase 1 — CAD (Fusion 360)

Design karo:

- Torso / Chassis

- Hip joint

- Upper leg

- Knee joint

- Lower leg

- Wheel + motor mounting

- Approximate mass & dimensions

## Phase 2 — MuJoCo Model (MJCF)

CAD geometry ko robot.xml mein convert karo. Define:

- Mass & Inertia

- Joints (hip, knee, wheel)

- Joint limits & damping

- Wheel radius & friction

- Ground + gravity

- Collision geometry

Example structure:

robot.xml

├── torso

├── left_leg  → hip → knee → wheel

└── right_leg → hip → knee → wheel

## Phase 3 — Pure Physics Test

Controller ke bina robot drop karo. Check:

- Gravity

- Collision

- Wheel-ground contact

- Friction

- Mass / Inertia

- Joint motion

Important: Physics galat hai to aage mat badho. AI mat lagana.

## Phase 4 — Motor Testing

Har actuator alag test karo:

- Hip motor → torque → leg moves

- Knee motor

- Wheel motor

## Phase 5 — Virtual Sensors

Add karo:

- IMU (pitch, pitch velocity)

- Encoders (joint angles + velocities)

- Contact sensors

## Phase 6 — Single Joint PID

Target angle set karke PID implement karo. Example:

Target angle = 30°    Actual angle = 20°    Error = 10°

PID motor torque calculate karega. Phir multi-joint pe apply karo.

## Phase 7 — Balance Controller

IMU → Pitch → Balance Controller → Wheel torque

Yahan two-wheeled inverted pendulum behaviour develop hoga.

## Phase 8 — Full Leg + Wheel Coordination

Balance + Hip/Knee configuration + Wheel movement ko ek saath coordinate karo.

## Phase 9 — Uneven Terrain + Disturbance

Flat ground → slopes → random terrain → external push / disturbance test karo.

## Phase 10 — Advanced Control + RL (Later)

LQR / MPC → PyTorch PPO → Domain Randomization → Isaac Lab → Sim-to-Real

# 5. Important Rules (Follow Strictly)

- RL se start mat karo. Pehle physics + PID/LQR solid banao.

- Isaac Sim / Isaac Lab abhi mat lagao. Jab MuJoCo mein balance + terrain handle ho jaye tab.

- ROS 2 day-1 requirement nahi hai. Real robot ke time lagana.

- URDF/MJCF software nahi hain — ye sirf robot description formats hain. MuJoCo ke liye initially MJCF prefer karo.

- Har phase ke baad data plot karo (Matplotlib): Pitch vs Time, Wheel speed vs Time, Joint angles, Torque, COM trajectory.

# 6. Software Installation Order (Abhi)

- Fusion 360

- Python 3.10+

- VS Code

- Git

- MuJoCo

- NumPy, SciPy, Matplotlib

- PyTorch + Gymnasium (baad mein)

# 7. Summary — Exact Path

Is order ko follow karoge to confusion nahi hoga aur progress measurable rahega:

CAD (Fusion 360)

→ MJCF Model

→ MuJoCo Physics Validation

→ Sensors

→ PID → Balance

→ Full Coordination

→ Uneven Terrain

→ LQR / MPC

→ RL (PPO)

→ Isaac Lab + Domain Randomization

→ Real Robot (ROS 2)

Note: Agar kisi phase ka detailed code, MJCF example, PID formula, ya LQR derivation chahiye to us phase ka full breakdown alag se bana sakte ho.